Device Assistant can write cover code without original screen and upgrade firmware. Please read the following carefully before using Device Assistant:

1. How to check whether your Multifunctional Recover Device supports Device Assistant?

(1) Power on or hold down the power button to turn on Multifunctional Recover Device

(2) Observe the version information displayed at the top of the screen. If the version date is 	earlier than 2022.04.12_11:11:07, you must use a U disk to upgrade; if the date is equal to 	2022.04.12_11:11:07, you can only use the device assistant to upgrade, do not use a U disk 	upgrade; If the date is later than 2022.04.12_11:11:07, Device Assistant or U disk upgrade is 	supported at the same time.

- How to upgrade firmware using a USB disk?

- Make sure the version date observed in step 1 is earlier than 2022.04.12_11:11:07

- Copy the file r100_update_mcu.bit and r100_update_mcu_app.bit from the software package to the root directory of the USB disk.

(3) Power off the Multifunctional Recover Device, insert the USB disk, and press the Read and Power keys to switch on the power and enter the upgrade mode.

Note: This step is to upgrade an earlier R100 version to the latest version to support Device Assistant. This step only needs to be performed once.

After that You can use the Device Assistant to upgrade on the PC.

3. How to install Device Assistant on the computer?

(1) Decompress the Device Assistant software package to a directory you specify, and go to the decompressed folder. Double-click the drv.exe file to install the serial port driver

(2) Double-click Device Assistant.exe to run the Device Assistant software

4. How to read the screen cover code through the Device Assistant?

(1) Connect Multifunctional Recover Device to computer with USB cable.

(2) Click the TrueTone Recover button of the device assistant to switch to the truetone recover function page.

(3) Connect the screen through the adapter board of Multifunctional Recover Device and wait for the Device Assistant to identify your screen model.

(4) Click the [Read] button. After reading, the cover code will be displayed in the edit box

5. How to write the cover code into the screen through the Device Assistant?

(4) Input your cover code into the edit box, and then click the [Write] button to write.

6. How to upgrade the firmware of the Multifunctional Recover Device using the Device Assistant?

(1) Connect Multifunctional Recover Device to computer with USB cable.

(2) Run the Device Assistant main program Device Assistant.exe and click Firmware Update to switch to the firmware upgrade function page.

(3) Click the [Open] button and select the r100_update_mcu_app.bit file in the current directory in the popup file selection dialog box

(4) Click the "Start" button to upgrade the firmware.

Note: Do not cut off the power supply of THE Multifunctional Recover Device or disconnect the USB connection during the upgrade. Otherwise, the Device may be damaged.

7. What should I do After the USB is plugged into the computer but the Device Assistant still prompts that the Multifunctional Recover Device cannot be connected?

Run the drv.exe file to install the serial port driver. When the system asks you whether to install the serial port driver, click OK

Right-click [My Computer]->[Properties]->[Device Manager]->[Ports (COM and LPT)] to confirm the presence of the USB-Serial CH340.

8.  What should I do if the Read/Write button is gray and cannot be clicked?

Ensure that the screen is properly connected to the Multifunctional Recover Device conversion board and the screen model can be correctly identified on the R100 screen.

9. What should I do if the USB disk is can’t recognized by Multifunctional Recover Device?

Please make sure that the format of your USB disk is FAT32. If not please reformat it to FAT32 on your computer.

10. How to get more help?

Please use wechat to scan the qr code below to add wechat official account: szdlzxkj